## FLASHING INSTRUCTION FOR NRF52840

There are 2 ways to do this either using J link flash software or nrf connect vscode.

### 1. USING J-FLASH: (J LINK ADAPTER):

- download segger J-Link from https://www.segger.com/downloads/jlink/

- Connect the nrf52840 through the J-link adapter to your PC .

| J-Link Pin | Signal Name | nRF52840 Pin |
| --- | --- | --- |
| 1 (VTref) | VCC (Target Voltage) | VDD (3.3V) |
| 4 (GND) | Ground | GND |
| 7 (SWDIO) | SWD Data | SWDIO |
| 9 (SWCLK) | SWD Clock | SWDCL |
| 15 (nRESET) [Optional] | Reset | P0.18 (RESET) |

- Open J-Link Flash and create a new project

- A prompt will open asking for target device , select nrf52480 as target device and leave the other setting as it is.

- now drag the merged.hex file from the (flashing_guide->bin) folder to the box asking for hex file or you can also select it from file->Open data file

- now navigate to target-> and press Connect

- now navigate to target->manual programming-> program and verify

- if it shows target programmed and verify successfully ,you can run the application by target->manual programming-> Start Application

Note: if the flashing fails , try to erase the chip by navigating and selecting target->manual programming-> Erase chip and then reprogramming it again.

# 2. USING NRF CONNECT SDK (VSCODE):

1 - Setup NRF connect SDK on VS Code

- VS Code Installation

  - Download the Visual Studio Code installer for Windows.

  - Once it is downloaded, run the installer (VSCodeUserSetup-{version}.exe). This will only take a minute.

  - Open VS Code.

- Add NRF Connect SDK extension in VS Code.

  - Add both the nrf connect sdk extensions.

  - After installing the extensions nrf connect extension icon will appear on the left bar.

Install Toolchain- lick on Manage toolchain, then install the nRF Connect SDK Toolchain v2.7.0.

-Install SDK

Then do the same for Manage SDKs, install v2.7.0

# 2 - Install J-Link tools

Download J-Link tools from here We used v7.94e. Download the latest from this link for the tools here

-	Follow the installation guide and install

# 3 - Flashing the Device

- Go to NRF Connect Extention Tab. Select “Add Build Configuration”.

- Select “prj.conf” as Base configuration files and “application.overlay” as Base Devicetree overlay.

- Click Build Configuration.

- Once build, the device can be flashed from the Actions section.

## MONITORING LOGS FOR NRF52840

### USING UART TO USB BRIDGE/ADAPTER (CP210X):

- install the CP210x driver using this link

- now plugin the adapter to your pc

- connect the txd pin of the adapter to the TRACE DATA 2 pin (p0.11)

- connect the rxd pin of the adapter to the TRACE DATA 1 pin (p0.12)

- now go to device manager->ports and see if the device is detected , if its already there then its okay if not then update driver and select the CP210x.zip file downloaded previously

- you will be able to monitor logs using any serial monitor and selecting the COM port at which Silicon Labs Cp210x is present and setting the baud rate to 115200 .

- Logs will only appear after the merged.hex  file is flashed and the NRF52840 is powered on.

On successfull setup following logs will be printed :(Note: if the logs are not showing press reset the nrf and do it while monitoring)

Scanning I2C0 bus...

Sensor detected at 0x76 Bus:I2C0

I2C0 scan complete.

Scanning I2C1 bus...

I2C1 scan complete.

ICM45686 found

Initializing ICM45686

Successfully data read from WHO_AM_I register

WHO_AM_I: 0x0

Buffer: 100000008000

Successfull data fetch from ACCEL_REG

Buffer2: 000000800000

Successfull data fetch from GYRO_REG

Accel: X=4096, Y=0, Z=-32768

Gyro: X=0, Y=128, Z=0

Buffer: 100000008000

Successfull data fetch from ACCEL_REG

Buffer2: 000000800000

Successfull data fetch from GYRO_REG

Accel: X=4096, Y=0, Z=-32768

Gyro: X=0, Y=128, Z=0

if the sensor fails to detect:

Scanning I2C0 bus...

Sensor detected at 0x76 Bus:I2C0

I2C0 scan complete.

Scanning I2C1 bus...

I2C1 scan complete.

ICM not found....